Project plan

This is a great decision. Starting from zero is the best way to ensure you actually understand the "why" behind the code, which is what will impress your professor and recruiters.

I have broken the project into 4 clear stages. We will focus on building a "Traffic Classifier"—an algorithm that sits on a network and flags "Attacker" traffic patterns.

Phase 1: The Environment (Week 1)

You need a "playground" where you can be both the hacker and the police.

- Operating System: Use Linux (Ubuntu is fine). If you are on Windows, install WSL2 or a Virtual Machine.

- Install the "Tor Daemon": This is the engine, not the browser.

  - Command: sudo apt install tor

- Install Chutney: This is a tool developed by the Tor Project to launch a private network on your own computer.

  - Download from: https://gitlab.torproject.org/tpo/core/chutney

  - Goal: Run ./chutney configure networks/basic and ./chutney start networks/basic.

  - Now you have your own private Darknet.

Phase 2: Data Collection (Week 2)

An algorithm is only as good as the data it learns from. You need to "record" what a hacker looks like.

- Capture Normal Traffic: * Open your terminal and use curl --socks5-hostname 127.0.0.1:9050 https://www.wikipedia.org.

  - Use Wireshark to record this. Save it as normal.pcap.

- Capture Attacker Traffic:

  - Run an aggressive script through Tor. For example, use nmap to scan a target (your own local server) via the Tor proxy.

  - Record this as attack.pcap.

- Observation: Look at the "Traffic Volume" in Wireshark. You will notice that the Attacker traffic has huge spikes and very specific packet sizes compared to the Normal traffic.

Phase 3: The Detection Algorithm (Week 3)

This is the "Logic" part of your project. We will use Python and the Stem library.

- Logic: You will write a script that "listens" to the Tor Control Port.

- Features to track:

  - Cell count: How many "cells" (fixed-size 512-byte packets) are moving per second?

  - Inter-arrival time: Is the time between packets very consistent (like a bot) or random (like a human)?

- The "Optimization": Instead of letting the attacker finish their task, your algorithm will detect the "Spiky" pattern within 10 seconds and "kill" the circuit.

Example Python Logic (Draft):

Python

from stem.control import Controller

with Controller.from_port(port = 9051) as controller:

controller.authenticate()

# Your code will monitor 'BW' (Bandwidth) events

# If bandwidth spikes > threshold, flag it as an attacker

Phase 4: The Final Presentation (Week 4)

To get that Letter of Recommendation, you need a polished demo.

- The "Before": Show the attacker successfully stealing data/scanning through Tor.

- The "After": Show your algorithm running. Within seconds of the attack starting, your console should flash "MALICIOUS ACTIVITY DETECTED - CIRCUIT CLOSED."

- The Metrics: Create a small table showing your "Detection Accuracy." (e.g., "Detected 9/10 attack simulations").